/boot partition is completely wiped out or corrupted in RHEL 7

Create an issue

Become root

cd /boot

ll

rm -rf *

ll

init 6

/boot partition is completely wiped out or corrupted then getting an error as shown in the image below

Attach dvd.iso as shown in the image below

Reset OS through VM

Press Enter on Troubleshooting mode

Press Enter on Rescue a Red Hat Enterprise Linux system (Rescue mode)

1) Continue 1 Enter

Enter

df -h

chroot /mnt/sysimage

df -h

cd /boot

ls

cd /

mount /dev/sr0 /mnt

cd /mnt

ls

cd Packages/

ls

pwd

uname -r

ls -ltr | grep -i kernel

rpm -ivh kernel-3.10.0-1160.e17.x86_64.rpm –force

cd /boot

ls

cd /mnt/Packages/

ls

ls -ltr | grep -i grub

rpm -ivh grub2-2.02-0.87.e17.x84_64.rpm --force

cfdisk /dev/sda	(To find the boot partition)

grub2-install /dev/sda1

cd /boot

ls

cd /mnt/Packages/

rpm -ivh redhat-logos-70.7.0-1.e17.noarch.rpm –force

(Press Tab key after rpm -ivh redhat-logos)

cd /boot

ls -ltr

cd grub2

ls -ltr

grub2-mkconfig -o /boot/grub2/grub.cfg

ls -ltr

exit

Remove dvd.iso as shown in the image below

Reset OS through VM

The system is started booting up without any issue